⚡ 1️⃣ Structure of 5-Level CHB Inverter

Each phase leg consists of two single-phase H-bridges connected in series:

(Vdc1) ─ [H-Bridge 1] ──┬── [H-Bridge 2] ─ (Vdc2)

│

Output

Each H-bridge has 4 switches (say S1–S4 for Bridge 1, S5–S8 for Bridge 2).

- Each H-bridge produces three possible output voltages:

  - +Vdc (top switches ON)

  - 0 (diagonal switches ON)

  - −Vdc (bottom switches ON)

When the two H-bridges’ outputs add up in series, the total output voltage has 5 levels:

Vout={−2Vdc,−Vdc,0,+Vdc,+2Vdc}V_{out} = \{-2V_{dc}, -V_{dc}, 0, +V_{dc}, +2V_{dc}\}Vout​={−2Vdc​,−Vdc​,0,+Vdc​,+2Vdc​}

⚙️ 2️⃣ Individual H-Bridge Operation

Each H-bridge can be represented by the table:

| H-Bridge Output | S1 | S2 | S3 | S4 | Output Description |
| --- | --- | --- | --- | --- | --- |
| +Vdc | 1 | 1 | 0 | 0 | Upper switches ON (positive terminal connected to +Vdc) |
| 0 | 0 | 1 | 1 | 0 | Diagonal conduction path (output shorted through middle switches) |
| −Vdc | 0 | 0 | 1 | 1 | Lower switches ON (output connected to −Vdc) |

These three modes give a clean transition between voltage levels and prevent shoot-through.

⚡ 3️⃣ Cascaded Combination (Two H-Bridges)

Now, by combining Bridge-1 and Bridge-2, we can form the 5 voltage levels as follows:

| Desired Output | Bridge 1 Output | Bridge 2 Output | Bridge 1 (S1–S4) | Bridge 2 (S5–S8) |
| --- | --- | --- | --- | --- |
| +2Vdc | +Vdc | +Vdc | 1 1 0 0 | 1 1 0 0 |
| +Vdc | +Vdc | 0 | 1 1 0 0 | 0 1 1 0 |
| 0 | 0 | 0 | 0 1 1 0 | 0 1 1 0 |
| −Vdc | −Vdc | 0 | 0 0 1 1 | 0 1 1 0 |
| −2Vdc | −Vdc | −Vdc | 0 0 1 1 | 0 0 1 1 |

🧠 4️⃣ Switching Logic Summary

- The sign of the reference level L decides which bridges are positive or negative.

- The magnitude of L decides how many bridges should output +Vdc or −Vdc.

For example:

- If L = 2: Both bridges are +Vdc → +2Vdc total

- If L = 1: One bridge is +Vdc, one is 0 → +Vdc total

- If L = 0: Both are 0 → 0V total

- If L = −1: One is −Vdc, one is 0 → −Vdc total

- If L = −2: Both are −Vdc → −2Vdc total