Topology & assumptions (clear and unambiguous)

I’ll assume a 3-module cascaded H-bridge per phase (common for 7-level), where each module can produce {+Vm,0,−Vm}\{+V_{m},0,-V_{m}\}{+Vm​,0,−Vm​}. All modules have equal DC source VmV_mVm​. Summing 3 modules yields 7 distinct levels:

Vph∈{−3Vm, −2Vm, −Vm, 0, +Vm, +2Vm, +3Vm}.

We use 12 gate signals:

- Module1: G1,G2,G3,G4

- Module2: G5,G6,G7,G8

- Module3: G9,G10,G11,G12

Switch numbering & canonical per-module mapping (used below):
For each module i with gates [GtopL, GbotL, GtopR, GbotR]:

- +V_m (module output = +V) → turn ON Top-Left and Bottom-Right (GtopL=1, GbotR=1).

- -V_m (module output = −V) → turn ON Bottom-Left and Top-Right (GbotL=1, GtopR=1).

- 0 (module zero) → choose Z+ (both top switches ON) or Z- (both bottom switches ON). We'll pick Z+ for clarity: (GtopL=1, GtopR=1).

IMPORTANT: complementary switches are the other half-bridge switches. In code use G_bot = 1 - G_top style for complements so you never set complementary devices ON simultaneously.

2) How each level is formed (conceptually)

Let module outputs be m1,m2,m3∈{+1,0,−1}m_1,m_2,m_3\in\{+1,0,-1\}m1​,m2​,m3​∈{+1,0,−1} where +1 → +V, 0 → 0, -1 → -V. Then:

- L=m1+m2+m3∈{−3,...,+3}L = m_1 + m_2 + m_3 \in\{-3,...,+3\}L=m1​+m2​+m3​∈{−3,...,+3}.
Examples:

- +3 → m=[+1,+1,+1]

- +2 → m=[+1,+1, 0] (or [+1,0,+1] alt)

- +1 → m=[+1,0,0] (or 0,+1,0 etc.)

- 0 → m=[0,0,0] (or +1,+1,-2 combos if available — but use module zeros)

- negative levels symmetric.

Multiple module combinations can realize the same L (redundancy). Use a balancing rule to alternate choices so modules share stress.

3) Truth table — canonical mapping (module-level → gate-level)

Below I give a clear canonical mapping (choose Z+ = both top ON for 0 state). Each module uses the per-module pattern explained above.

Legend per module:

- Module1 gates: [G1,G2,G3,G4] → G1 = top-left, G2 = bottom-left, G3 = top-right, G4 = bottom-right

- Module2 gates: [G5,G6,G7,G8]

- Module3 gates: [G9,G10,G11,G12]

Module mapping rules (again):

- +V → (top-left ON, bottom-right ON) → pattern: [1,0,0,1]

- 0 (Z+) → (top-left ON, top-right ON) → [1,0,1,0]

- -V → (bottom-left ON, top-right ON) → [0,1,1,0]

Now the canonical gate-patterns for level L:

| L | m1 | m2 | m3 | G1 G2 G3 G4 | G5 G6 G7 G8 | G9 G10 G11 G12 |
| --- | --- | --- | --- | --- | --- | --- |
| +3 | +1 | +1 | +1 | 1 0 0 1 | 1 0 0 1 | 1 0 0 1 |
| +2 | +1 | +1 | 0 | 1 0 0 1 | 1 0 0 1 | 1 0 1 0 |
| +1 | +1 | 0 | 0 | 1 0 0 1 | 1 0 1 0 | 1 0 1 0 |
| 0 | 0 | 0 | 0 | 1 0 1 0 | 1 0 1 0 | 1 0 1 0 |
| -1 | -1 | 0 | 0 | 0 1 1 0 | 1 0 1 0 | 1 0 1 0 |
| -2 | -1 | -1 | 0 | 0 1 1 0 | 0 1 1 0 | 1 0 1 0 |
| -3 | -1 | -1 | -1 | 0 1 1 0 | 0 1 1 0 | 0 1 1 0 |

Notes:

- The table uses the canonical decomposition [+1,+1,0] for +2 etc. You can choose alternate decompositions to balance switching losses (e.g., sometimes use [+1,0,+1]).

- The chosen 0 state is Z+ for each module (all top switches ON). If you prefer Z− (all bottom ON), replace module zero pattern with [0,1,0,1] consistently.