Step 1: Concept Recap — 9-Level CHB Structure

A 9-level inverter requires 4 H-bridges in series (per phase).
Each H-bridge can output three levels:

+Vdc,  0,  −Vdc\text{+Vdc}, \; 0, \; -Vdc+Vdc,0,−Vdc

So the total possible levels:

{−4,−3,−2,−1,0,+1,+2,+3,+4}\{-4, -3, -2, -1, 0, +1, +2, +3, +4\}{−4,−3,−2,−1,0,+1,+2,+3,+4}

Each level is obtained by the combination of the 4 H-bridge outputs.
Total gate signals = 4 × 4 = 16 gates per phase.

🔹 Step 2: Truth Table for 9-Level CHB (4 H-bridges)

| Output Level (L) | Bridge-1 | Bridge-2 | Bridge-3 | Bridge-4 | Description |
| --- | --- | --- | --- | --- | --- |
| +4 | +Vdc | +Vdc | +Vdc | +Vdc | All positive |
| +3 | +Vdc | +Vdc | +Vdc | 0 | Top 3 positive |
| +2 | +Vdc | +Vdc | 0 | 0 | Top 2 positive |
| +1 | +Vdc | 0 | 0 | 0 | One positive |
| 0 | 0 | 0 | 0 | 0 | All zero |
| -1 | -Vdc | 0 | 0 | 0 | One negative |
| -2 | -Vdc | -Vdc | 0 | 0 | Two negative |
| -3 | -Vdc | -Vdc | -Vdc | 0 | Three negative |
| -4 | -Vdc | -Vdc | -Vdc | -Vdc | All negative |

🔹 Step 3: Gate Logic per H-bridge

Each H-bridge uses 4 switches (S1–S4) and follows this pattern:

| Bridge Output | S1 | S2 | S3 | S4 |
| --- | --- | --- | --- | --- |
| +Vdc | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| -Vdc | 0 | 0 | 1 | 1 |