Multilevel Inverter Design and Control: A Comprehensive Report

1. Introduction to Multilevel Inverters

1.1. Defining Multilevel Inverters: Concept and Purpose

Multilevel inverters (MLIs) represent a significant advancement in DC-AC power conversion technology, moving beyond the binary (two-level) voltage output of conventional inverters. Historically, traditional inverters were limited to generating output voltages that switched between two discrete levels, typically 0V and a DC bus voltage (Vdc), resulting in a square-wave output.1 This inherent limitation posed challenges for high-power applications, where such waveforms often led to poor power quality and high voltage stress on components.1

In response to these limitations, the concept of multilevel inverters emerged in the late 20th century, marked by the introduction of three-level converters.1 This development heralded a "new generation" of DC-AC converters specifically designed for medium and high voltage and power levels.3 The fundamental principle of MLIs involves synthesizing a desired AC output voltage by utilizing multiple discrete DC voltage levels. These levels are typically derived from capacitor voltage sources or an array of multiple DC links, creating a staircase-like approximation of a sinusoidal waveform.1 This multi-level approach allows for a finer resolution in the output voltage waveform, enabling a much closer approximation of a pure sine wave. This capability is critical for modern power systems that demand cleaner power, higher efficiency, and compatibility with sensitive loads and stringent grid codes. The ability to generate multiple voltage levels directly leads to an output waveform that inherently contains significantly lower harmonic content, minimizing electromagnetic interference (EMI) and reducing the rate of voltage change (dv/dt) across switches.1 This reduction in distortion means less stress on connected loads, improved efficiency, and often eliminates the need for bulky and expensive output filters.6

1.2. Advantages over Conventional Two-Level Inverters

Multilevel inverters offer a compelling set of advantages that position them as a superior alternative to traditional two-level inverters for a wide array of applications. These benefits collectively address the inherent limitations of conventional designs, leading to superior performance and reliability.

A primary benefit of MLIs is their capability to produce output voltage and current waveforms with significantly lower harmonic content and reduced Total Harmonic Distortion (THD).1 This makes the output much closer to a pure sinusoidal waveform, a critical factor for power quality. This improved waveform quality often eliminates the need for bulky and expensive filters and transformers, simplifying the overall system design and reducing cost.6

Furthermore, MLIs inherently generate almost no electromagnetic interference (EMI) and smaller common-mode (CM) voltage.1 This characteristic is particularly beneficial in applications involving motor drives, as it reduces stress on motor bearings and improves the electromagnetic compatibility of the entire system. The ability to generate multiple voltage levels allows for a lower rate of voltage change (dv/dt) across switches, which directly contributes to reduced switching losses and EMI.8

MLIs achieve higher overall efficiency and significantly reduced switching losses compared to two-level inverters by distributing the total voltage stress across multiple semiconductor devices and operating at lower effective switching frequencies.1 This can lead to substantial energy savings, with some studies indicating a 60% reduction in total rated load losses with a five-level inverter.9 The distributed voltage stress across individual switches also minimizes the dv/dt impact, contributing directly to enhanced reliability, longer device lifespan, and improved thermal management of the power semiconductor devices.2

MLIs are exceptionally well-suited for demanding applications requiring high power and high voltage operation.2 They can generate higher output voltages without necessitating the series connection of high-voltage-rated switching devices, as the voltage stress is effectively distributed among multiple lower-rated components.9 While MLIs can involve a higher component count, the ability to eliminate bulky filters and transformers, coupled with improved efficiency and reduced stress on components, can lead to overall system cost-effectiveness and simplified design in certain high-power applications.6

Certain MLI topologies, notably the Cascaded H-Bridge, offer inherent modularity and flexibility.1 This characteristic makes them highly scalable for higher power levels and facilitates easier manufacturing, maintenance, and fault isolation in large-scale systems. Furthermore, MLIs are particularly adept at interfacing with various renewable energy sources (such as solar photovoltaic arrays and wind turbines), plug-in electric vehicles, battery energy storage systems, and capacitors.1 Their design helps optimize energy conversion from these DC-based sources to the AC grid. This capability positions MLIs as a critical enabling technology for the evolving landscape of modern power systems, directly aligning with the stringent requirements of modern grid codes, the need for efficient energy management in EVs, and the reliable integration of intermittent renewable sources.

In contrast, conventional two-level inverters are limited in high-power applications, suffer from high voltage stress, operate with lower efficiency, and generate significant harmonic distortion, often requiring bulky filters and operating at high switching frequencies that lead to substantial losses.7 The very mechanism that enables MLIs to achieve superior performance—generating more voltage levels—inherently requires a greater number of power semiconductor switches, associated gate driver circuits, and often more passive components (diodes, capacitors).6

This directly increases the bill of materials, manufacturing complexity, physical size, and potential points of failure. This establishes a core design challenge in the field of MLIs: how to maximize the performance benefits (e.g., waveform quality, efficiency, voltage capability) while simultaneously minimizing the "cost" in terms of component count, physical footprint, and overall control complexity. This fundamental tension is a major driving force behind a significant portion of current research, particularly the development of "reduced switch count" (RSC) and hybrid topologies 14, which aim to achieve high performance with fewer components, albeit sometimes at the expense of other factors like modularity or control intricacy.

Table 1: Comparison of Multilevel Inverters vs. Two-Level Inverters

| Characteristic | Conventional Two-Level Inverter | Multilevel Inverter (MLI) |
| --- | --- | --- |
| Number of Output Voltage Levels | 2 (0, Vdc) | >2 (3, 5, 7, up to 125 levels mentioned) |
| Output Waveform Quality (THD) | High THD, requires bulky filters | Low THD, near-sinusoidal, often eliminates filters 1 |
| Electromagnetic Interference (EMI) / Common-Mode Voltage | Higher EMI, significant common-mode voltage | Lower EMI, reduced common-mode voltage 1 |
| Efficiency / Switching Losses | Lower efficiency, high switching losses at high frequencies | Higher efficiency, reduced switching losses due to lower switching frequency and distributed stress 1 |
| Voltage Stress on Switches | High voltage stress on individual devices | Reduced voltage stress on individual devices 2 |
| Suitability for High Power/Voltage Applications | Limited for high power/voltage | Well-suited, high power capability 2 |
| Component Count / Complexity | Simpler circuit, fewer components | Higher component count (switches, diodes, capacitors), more complex gate drivers 1 |
| Interfacing with RES/EVs | Less optimized | Easily interfaced, optimizes energy conversion 1 |

2. Key Multilevel Inverter Topologies and Their Operational Principles

Multilevel inverters are broadly categorized into several classical topologies, each with distinct operational principles, advantages, and disadvantages. The three most commonly used and foundational topologies are the Diode-Clamped Multilevel Inverter (DCMLI), the Flying Capacitor Multilevel Inverter (FCMLI), and the Cascaded H-Bridge Multilevel Inverter (CHB-MLI).4

2.1. Diode-Clamped Multilevel Inverters (DCMLI)

The Diode-Clamped Multilevel Inverter (DCMLI), also known as a Neutral-Point Clamped (NPC) inverter, is a widely adopted topology, particularly in medium voltage applications.

- Basic Principle and Circuit Overview: The fundamental principle of DCMLI involves using diodes as clamping devices to limit the voltage stress across individual power semiconductor switches and to achieve stepped output voltages. In this topology, the DC bus voltage is typically divided into multiple levels by a series of capacitors. The diodes then "clamp" the voltage across each switch to a specific fraction of the total DC bus voltage.

For an n-level inverter, the design typically requires (n-1) DC-link capacitors, 2(n-1) switching devices (e.g., IGBTs or MOSFETs), and (n-1)(n-2) clamping diodes. The voltage across each capacitor and each switch is ideally Vdc/(n-1). By increasing the number of voltage levels, the quality of the output voltage is improved, becoming closer to a sinusoidal waveform.

- Example (Three-Level DCMLI): A three-level DCMLI (n=3) utilizes two capacitors (C1, C2) on the DC bus, four switches (S1, S2, S1', S2'), and two clamping diodes. With a total DC bus voltage of Vdc, each capacitor has Vdc/2 across it, and each device's voltage stress is limited to Vdc/2. The output voltage can synthesize three levels: +Vdc/2 (S1, S2 ON), 0 (S2, S1' ON), and -Vdc/2 (S1', S2' ON).

- Example (Five-Level DCMLI): For a five-level DCMLI (n=5), four capacitors (C1-C4) are used, resulting in Vdc/4 across each. It requires eight switches and six clamping diodes. The output voltage can synthesize five levels: +Vdc/2, +Vdc/4, 0, -Vdc/4, and -Vdc/2. To achieve Vdc/2 output, four switches must conduct simultaneously.

- Advantages: DCMLIs offer several significant advantages, including the ability to control both real and reactive power, often eliminating the need for additional filters to reduce harmonics. The distributed voltage stress across switches leads to enhanced reliability. They are known for high power efficiency, simplicity in their basic configuration, and suitability for high voltage systems and variable speed motor drives.

- Disadvantages: A notable drawback of the diode-clamped topology is that the maximum output voltage is typically limited to half of the total DC source voltage. This issue can be mitigated by using a two-times voltage source or by cascading multiple DCMLIs.5 This voltage utilization limitation implies that while the topology is effective, it might require additional complexity or components to achieve desired output voltage ranges, which could impact overall system cost and efficiency. Furthermore, the number of clamping diodes increases quadratically with the number of levels, adding to the system's complexity and cost for higher level counts.5 This rapid increase in component count for higher levels implies a significant trade-off: while DCMLIs offer good performance, their scalability for very high voltage levels (requiring many levels) becomes problematic due to increased complexity, cost, and potential reliability issues from the sheer number of diodes. Challenges also include difficulties with actual power flow due to potential undercharging or overcharging of the DC levels, and the ongoing need to stabilize the DC link capacitance voltage.

2.2. Flying Capacitor Multilevel Inverters (FCMLI)

The Flying Capacitor Multilevel Inverter (FCMLI), also known as a capacitor-clamped inverter, utilizes floating capacitors to synthesize multiple voltage levels.

- Basic Principle and Circuit Overview: Similar to DCMLIs, FCMLIs generate stepped output voltages, but they employ a ladder-like configuration of floating capacitors to restrict voltage levels rather than clamping diodes.6 Each phase leg in an FCMLI consists of multiple capacitors (typically three or more) connected between the DC source and power switches.18 These capacitors dynamically distribute the voltage among themselves to generate the desired output waveform.18 A unique feature of FCMLIs is the ability to extend the number of voltage levels by changing the ratio of the capacitor voltages.15 For instance, a three-cell FCMLI traditionally yields four levels with a 1:2:3 capacitor voltage ratio, but altering this ratio can enable operation as a five-, six-, seven-, or even eight-level inverter.15 The eight-level case, referred to as "maximally distended," utilizes all possible transistor switching states but may lack sufficient per-phase redundancy for capacitor voltage balancing.15 Some proposed FCMLI topologies can operate with a single independent DC source, along with multiple capacitors, diodes, and switching devices.15

- Capacitor Voltage Balancing Mechanisms: A critical aspect of FCMLI operation is maintaining the voltage balance of the floating capacitors.16 These capacitor voltages must be precisely controlled and maintained at well-defined levels to ensure correct operation of the multilevel inverter.16 This challenge of capacitor voltage balancing is a central research area for FCMLIs, as the inherent nature of floating capacitors necessitates sophisticated control algorithms and redundancy schemes to ensure stable and reliable operation.17 Research in this area focuses on various capacitor voltage balancing methods, including:

- Switching State Selection: Strategies that prioritize the selection of switching states based on the preferential charging and discharging needs of the capacitors.17

- Comparator-Based Control: Utilizing two-level or three-level comparators for capacitor state detection and control.17

- Sampling Frequency-Based Approaches: Techniques that control capacitor voltages based on sampling frequency for improved performance.17

- Redundancy Schemes: Employing redundancy involving all phases to improve capacitor voltage balancing.18 Some newer topologies aim for self-balanced capacitor voltages without additional complex control algorithms.19

- Soft Charging: Research explores soft charging mechanisms for capacitors to reduce losses during charging and mitigate high inrush currents.14

- Advantages: FCMLIs offer greater flexibility in voltage synthesis compared to DCMLIs and can manage both actual and reactive power.13 Some proposed topologies require a single DC source and fewer clamping diodes than DCMLIs, leading to less THD, low power loss, and suitability for high power applications.15 They are also suitable for motor drive operation and reactive power transfer.18

- Disadvantages: A significant challenge for FCMLIs is the large number of capacitors required, which directly increases the cost and complexity of the system.13 This can also lead to high switching frequency losses during real power transfer.13 This highlights a trade-off: reducing one type of component (diodes, as in DCMLIs) increases another (capacitors), which can still lead to significant practical challenges in terms of bulk, cost, and the aforementioned balancing issues. Furthermore, the problem of high inrush currents during capacitor charging remains a concern 18, potentially increasing stress on switches or requiring additional protective circuitry. Long discharging periods and short charging windows can also lead to high voltage ripples, necessitating large and expensive capacitors.14

2.3. Cascaded H-Bridge Multilevel Inverters (CHB-MLI)

The Cascaded H-Bridge Multilevel Inverter (CHB-MLI) is distinguished by its modular and flexible architecture, making it a promising topology for high-power applications.

- Basic Principle, Modularity, and Isolated DC Sources: A CHB-MLI is constructed by connecting multiple single-phase H-bridge inverter cells in series on the AC side.2 Each individual H-bridge cell is typically supplied by its own isolated DC voltage source.2 Each H-bridge module can generate three voltage outputs: +Vdc, 0, or -Vdc.11 By summing the outputs of these series-connected H-bridges, a staircase output voltage waveform with multiple levels is created. For an
m-level inverter, the total number of levels in normal operation is 2k+1, where k is the number of H-bridge modules per phase.11 This modularity allows for easy scalability and simplifies maintenance, as individual modules can be added, removed, or bypassed.1 This represents a significant design paradigm shift compared to DCMLI or FCMLI, where the entire structure is more integrated, offering advantages in scalability, maintenance, and manufacturing.

- Example (Three-Phase 5-Level CHMI): For a three-phase 5-level CHMI, two H-bridge cells are needed per phase, totaling six H-bridge cells and 24 power switches for the entire circuit.2

- Advantages: The primary advantages of CHB-MLIs stem from their modularity and flexibility.1 This modular design allows for a straightforward increase in output voltage levels by simply adding more H-bridge cells. CHB-MLIs are capable of generating a pure sinusoidal waveform with significantly reduced levels of harmonics.3 They are well-suited for medium power output voltages with low voltage configuration of components.3 Furthermore, they can be designed as transformer-less systems, which reduces weight and bulk.13

- Disadvantages: The main drawback of CHB-MLIs is the requirement for separate, isolated DC sources for each H-bridge cell.2 This can increase the overall system cost and complexity, especially when many levels are desired. This requirement for isolated DC sources is a double-edged sword: while it enables modularity, it adds significant cost and complexity, and can lead to voltage imbalance issues. A significant challenge arises from DC voltage inequality or uneven power distribution among the H-bridge cells on the same phase, which can lead to power imbalance and voltage distortion at the AC side.11 This inter-bridge power imbalance can cause DC-link voltage drift, while inter-phase power imbalance leads to unbalanced grid current, which is often not allowed by grid standards.11

- Control Schemes: Various modulation techniques are employed for CHB-MLIs to achieve desired output quality and manage operational challenges:

- Multi-Carrier Sinusoidal Pulse Width Modulation (MSPWM): This technique involves comparing a sinusoidal modulation signal with several triangular carrier signals to generate gating signals.2 MSPWM can be categorized into Carrier Disposition (CD) methods (Phase Disposition (PD), Phase Opposition Disposition (POD), Alternative Phase Opposition Disposition (APOD)) and Phase Shifted (PS) methods.2

- Selective Harmonic Elimination-Pulse Width Modulation (SHE-PWM): This strategy is used to eliminate undesired low-order harmonics by precisely determining switching angles, often using optimization algorithms like Particle Swarm Optimization (PSO).20

- Fault-Tolerant Control: Given the potential for power imbalance and cell failures, fault-tolerant control is a critical research area for CHB-MLIs.11 Methods like the neutral-shift method 11 and Neutral Voltage Modulation (NVM) 23 are proposed to enable reliable operation even with damaged or reduced-voltage DC source modules. These methods aim to maintain balanced three-phase output with low THD by dynamically adjusting modulating signals and injecting common-mode voltage.23 This highlights that architectural choices have profound implications for control system design, particularly in managing the complexities introduced by multiple isolated sources.

Table 2: Comparative Summary of Major MLI Topologies

| Characteristic | Diode-Clamped (DCMLI) | Flying Capacitor (FCMLI) | Cascaded H-Bridge (CHB-MLI) |
| --- | --- | --- | --- |
| Key Components (per phase, for n levels) | (n-1) capacitors, 2(n-1) switches, (n-1)(n-2) clamping diodes 5 | (n-1) capacitors (floating), 2(n-1) switches, 0 clamping diodes 13 | (n-1)/2 H-bridges, (n-1)/2 isolated DC sources, 2(n-1) switches 2 |
| DC Source Requirement | Single DC bus with split capacitors 5 | Single DC source 15 | Multiple isolated DC sources 2 |
| Capacitor Voltage Balancing | Requires balancing of DC-link capacitors 13 | Critical and complex due to floating capacitors, active research area 16 | Not applicable in the same way, but DC source imbalance is an issue 11 |
| Modularity | Low | Moderate | High 1 |
| Complexity | Moderate (increases with levels due to diodes) | High (due to capacitor balancing and number of capacitors) | Moderate to High (due to multiple isolated DC sources and fault-tolerant control) |
| Typical Applications | Medium voltage applications, motor drives, variable speed motor drives 6 | Motor drive operation, reactive power transfer 18 | All-electric/hybrid-electric vehicle motor drives, large-scale grid-connected PV systems 10 |

3. Applications of Multilevel Inverters

The inherent advantages of multilevel inverters—such as high power capability, reduced harmonic distortion, high efficiency, and lower electromagnetic interference—make them exceptionally well-suited for a diverse array of demanding applications across various industries.

3.1. Electric Vehicles (EVs) and Hybrid Electric Vehicles (HEVs)

Multilevel inverters are a cornerstone technology for the propulsion systems of all-electric vehicles (EVs) and hybrid-electric vehicles (HEVs). They are primarily used in motor drives for traction, managing the electric motor that generates mechanical energy for propulsion.8

- Benefits in EVs/HEVs: MLIs enable the generation of near-sinusoidal voltages with fundamental frequency switching, significantly reducing electromagnetic interference (EMI) and common-mode voltage.10 This reduction in EMI contributes to a safer and more accessible vehicle environment, and lower common-mode voltage reduces stress on motor bearings, enhancing motor longevity.5 MLIs optimize energy conversion by increasing efficiency, which is crucial for extending vehicle range and overall performance.8 Their control flexibility allows them to offer high performance over wide voltage and frequency ranges, which is essential for handling the rapid changes in speed and loading characteristic of EV traction drives.8 This positions MLIs as a key enabler for improved EV performance and safety, directly impacting consumer adoption and regulatory compliance in the automotive industry.

- Topology Considerations: While both diode-clamped and cascaded H-bridge inverters are presented as applications for EV/HEV motor drives 10, specific challenges in EV applications (such as rapid changes in speed and loading, and power losses with varying modulation index and power factor) lead to preferences for certain topologies. For instance, 3-level Neutral-Point Clamped (NPC) and 3-level T-type Neutral-Point Clamped (TNPC) inverters are considered effective alternatives for EV applications, whereas traditional Cascaded H-bridge (CHB) and Flying Capacitor (FC) topologies might face specific problems.8 This illustrates that topology selection is highly dependent on application-specific constraints, requiring a deep understanding of the unique demands of the system.

3.2. Renewable Energy Systems (Solar PV, Wind Power)

Multilevel inverters play an indispensable role in the integration of renewable energy systems (RES), such as solar photovoltaic (PV) and wind power, into the electrical grid. They serve as crucial interfaces, converting the DC electricity generated by RES into high-quality AC power suitable for grid connection.4

- Role in RES Integration: MLIs help optimize energy conversion by increasing efficiency in renewable energy systems.8 Their ability to produce a purely sinusoidal waveform with fewer harmonics is vital for grid-tied systems, as grid codes demand high power quality and minimal harmonic injection.13 MLIs are particularly well-suited for large-scale RES integration.13 Research also explores novel MLI configurations for PV applications, including those with integrated DC-DC converters and switched-capacitor based designs.18 This positions MLIs as a fundamental bridge to a renewable energy future, enabling the efficient and high-quality integration of variable DC power sources into the AC grid.

- Challenges and MLI Solutions: The intermittent nature of RES, particularly PV sources, can lead to inconsistent power generation and voltage variations, necessitating robust MLI solutions for a steady energy supply.13 Issues like non-coordinated regulation of multiple PV sources and common-mode problems in transformer-less systems are also challenges that MLI design and control aim to address.13 This indicates that the role of MLIs extends beyond simple DC-AC conversion; they must also be designed and controlled to mitigate the inherent variability and grid interaction issues of RES. This creates a feedback loop: the challenges of RES integration drive further research and development in MLI topologies and control strategies (e.g., advanced MPPT, fault tolerance, smart grid integration) to make the overall system more robust and reliable.

3.3. Motor Drives and Industrial Applications

Multilevel inverters are extensively utilized in various industrial applications, particularly in high-power motor drives, due to their ability to deliver high-quality power and operate efficiently across a wide range of conditions.4

- Benefits in Industrial Settings: MLIs optimize energy conversion by increasing efficiency in industrial applications.8 Their capacity for high performance over wide voltage and frequency ranges makes them ideal for controlling electric motors, which often experience dynamic loads and speed variations.8 The improved quality of the output voltage and high efficiency of MLIs have led to increased industrial adoption.16 They are specifically used for high power drives.24 This implies that MLIs are not just about power conversion but are crucial for improving the overall efficiency and reliability of industrial processes that rely on electric motors. By providing cleaner power (reduced harmonics) and reducing stress on motors (lower common-mode voltage5), MLIs contribute to longer equipment lifespan, reduced maintenance, and ultimately, reduced operational costs, providing a direct economic benefit to industries.

- Topology Suitability: Diode-clamped MLIs are frequently employed in medium voltage applications for motor drives, often coupled directly to the grid without bulky transformers.6 Flying capacitor MLIs are also noted as suitable for motor drive operation.18

3.4. Uninterruptible Power Supplies (UPS)

Multilevel inverters are also employed in Uninterruptible Power Supply (UPS) systems, leveraging their inherent advantages to provide reliable and high-quality backup power.

- Role in UPS: In UPS applications, MLIs help optimize energy conversion by increasing efficiency.8 Their ability to deliver a high-quality, low-distortion output waveform is critical for protecting sensitive loads from power disturbances and ensuring continuous operation during grid outages.7 This highlights their role in ensuring power reliability and continuity, which is paramount for critical infrastructure such as data centers and medical facilities.

3.5. Medium/High Voltage Grid-Connected Systems

Multilevel inverters are increasingly popular for high-power applications in medium and high voltage grid-connected systems, where they offer significant advantages over conventional solutions.

- Direct Grid Interfacing: MLIs are specifically designed for medium and high voltage and power applications.3 In medium voltage applications, diode-clamped MLIs are often used to eliminate bulky transformers, allowing them to be coupled directly to the grid.6 This direct coupling reduces system size, weight, and losses associated with transformers, offering a direct pathway to higher efficiency and reduced system cost and footprint in grid-connected applications.

- Power Quality and Grid Stability: MLIs are becoming popular due to their improved harmonic profile and increased power ratings.2 They can inherently create high voltage and reduce harmonics through their circuit topology.2 For grid-tied systems, MLIs are crucial for injecting a purely sinusoidal waveform into the grid, meeting stringent grid codes regarding harmonic content and power factor.13 This positions MLIs as fundamental components of the entire power system infrastructure, suitable for large-scale power transmission and distribution, especially as grids become more decentralized with renewable energy.

- System Integration: The comprehensive review of MLIs for grid-tied systems discusses their principles, various topologies, modulation techniques, and control methods, emphasizing their role in integrating renewable energy sources into the main power supply.13 This indicates that MLIs are not static components but are actively evolving to meet the demands of modern, dynamic grids. Their ability to inject high-quality power and their increasing modularity 13 make them central to grid modernization efforts, including distributed generation and smart grid functionalities.

- Challenges in Grid Integration: Despite their benefits, grid-connected MLIs face challenges, including increased complexity with higher output levels.13 There is an ongoing need for further evaluation of new topologies for grid-integrated applications, and modular MLIs still face technical challenges in modeling, control, and protection within large-scale RES integration scenarios.13

4. Areas of Advancement and Research in Multilevel Inverters

The field of multilevel inverters is dynamic, with continuous research and development aimed at addressing existing limitations and expanding their capabilities. Key areas of advancement focus on novel topologies, sophisticated control strategies, enhanced reliability, and seamless integration into evolving power systems.

4.1. Novel Topologies: Reduced Switch Count (RSC) and Hybrid Configurations (Symmetrical vs. Asymmetrical)

Classical MLI topologies (NPC, FC, CHB) often require a high number of components as the number of output levels increases, leading to high cost and complex implementation.14 This "component count paradox"—where achieving better performance requires more components, which in turn increases cost and complexity—is the primary driver for extensive research into "reduced component count (RCC)" or "reduced switch count (RSC)" and hybrid topologies.4 The overarching aim is to achieve high power quality at reduced cost and complexity.14

- Symmetrical vs. Asymmetrical Configurations:

- Symmetrical MLIs: These topologies utilize identical DC voltage sources 4 and are often praised for their high modularity.1

- Asymmetrical MLIs: These employ DC voltage sources of unequal magnitude.4 Asymmetry is widely adopted in new RSC topologies due to their superior characteristics, such as higher efficiency, better switch utilization, and lower THD.14 They can generate many voltage levels with fewer power sources and components compared to symmetrical designs.1 This approach directly addresses the component count paradox by maximizing the number of output levels from a given set of components.

- Examples of Novel Topologies: Research has proposed new topologies with significantly fewer switch requirements, such as synthesizing 15 levels with 10 switches or 25 levels with 12 switches, offering reduced voltage stresses.4 Asymmetric cascaded half H-bridge MLIs have been developed that eliminate the need for polarity generation circuits, thereby reducing the number of switches requiring high blocking voltage.19 Hybrid topologies, which combine different MLI structures (e.g., switched capacitor circuits with conventional H-bridges) or integrate DC-DC converters, are also emerging to achieve high voltage gain and multiple levels with fewer components.1 Some designs focus on single DC source topologies with switched capacitors to achieve voltage gain and level generation, such as quadruple boost nine-level switched capacitor inverters for small-scale PV systems, which achieve self-balanced capacitor voltage with minimal components.14

- Challenges in Novel Topologies: While these new topologies aim to reduce component count, they must carefully avoid introducing excessive complexity or relying on too many different voltage sources.14 Issues like unbalanced power distribution, high inrush currents, and voltage ripples in capacitors remain challenges that require further investigation.14 The pursuit of RSC and hybrid topologies is not merely an academic exercise; it's a direct response to the economic and practical realities of mass production and deployment, as the success of MLIs in emerging markets depends heavily on achieving high performance at a competitive cost and footprint.

4.2. Advanced Control and Modulation Strategies

The precise control and modulation of multilevel inverters are paramount to unlocking their full performance potential. These strategies aim to achieve high-quality output voltage, eliminate lower-order harmonics, reduce switching losses, and improve overall system performance.6 The increasing complexity of MLI applications (e.g., grid integration, fault tolerance) necessitates equally sophisticated and adaptive control algorithms.

- Pulse Width Modulation (PWM) Variations: PWM remains a fundamental modulation technique. High Switching Frequency (HSF) methods include various carrier-based PWM (MCB-PWM) schemes like Phase Shift Carriers (PSC)-PWM, Level Shift (LS)-PWM, and Hybrid-PWM.13 Reference-based PWM techniques such as sinusoidal PWM (SPWM) are commonly used for simulations.5 LS-PWM can also be modified to enhance fault tolerance.23

- Selective Harmonic Elimination (SHE): This technique is specifically designed to eliminate certain low-order harmonic components by precisely calculating switching angles.19 It often involves solving non-linear transcendental equations, sometimes using optimization algorithms like Particle Swarm Optimization (PSO).20 SHE allows for low-frequency switching, which can significantly reduce unwanted harmonics.21 However, challenges exist in effectively eliminating harmonics at lower modulation indices.21

- Space Vector Modulation (SVM): SVM is a promising digital modulating technique that offers great flexibility in optimizing switching pattern design and is well-suited for digital implementation.22 It uses vector control signals to generate reference and output voltage waveforms, regulating output voltages according to switching conditions.13 SVM can reduce common-mode voltages and switching losses.22 While powerful, SVM can be difficult to control regarding sector selection and switching sequence design.22 Advanced SVM algorithms are being developed for various MLI topologies, including trans-Z-source inverters.24

- Model Predictive Control (MPC): MPC is a modern control technique utilized to predict future variable values and compute cost functions for optimal operation.13 It can handle control variables, input/output delays, and unstable systems, offering quick dynamic response.13 Modified MPCC algorithms are designed for MLIs to achieve low grid current ripples, effective capacitor voltage balancing, and reduced switch count.13

- Nearest Level Control (NLC): NLC is a non-carrier method that finds the nearest voltage level to a sinusoidal reference. It produces better voltage waveforms and reduces output/load current ripple, especially for high voltages, while minimizing switching events.13 However, it is not suitable for lower output voltage levels at low modulation indices due to high harmonic distortion.13

The pursuit of higher power quality and efficiency in MLIs necessitates increased control complexity. This is evident as advanced modulation techniques become more intricate with increasing voltage levels 13 and SVM presents control challenges.22 The very act of transforming the raw stepped output of an MLI into a high-quality, grid-compatible AC signal requires sophisticated algorithms that precisely shape the waveform, minimize distortion, manage switching losses, and even handle fault conditions. This area remains a fertile ground for research, particularly in combining techniques and leveraging computational power for real-time optimization. The use of optimization algorithms like PSO and predictive control methods like MPC also indicates a growing trend towards more intelligent and adaptive control strategies, moving beyond traditional control theory to handle non-linearities and optimize multiple objectives simultaneously.

4.3. Enhanced Capacitor Voltage Balancing Techniques

Maintaining stable capacitor voltages is crucial for the stable and reliable operation of MLIs, particularly for Flying Capacitor MLIs (FCMLIs) and some hybrid topologies, as unbalanced capacitor voltages can lead to voltage distortion and system instability.13 This is a major bottleneck for MLI performance and reliability; if capacitors are not balanced, the output waveform quality degrades, and components can be overstressed, potentially leading to system failure.

- Techniques: Research efforts focus on various strategies to ensure capacitor voltage balance:

- Switching State Selection: This involves selecting optimal switching states based on the preferential charging and discharging needs of the capacitors.17

- Multi-level Comparators: Using advanced comparators for better control of capacitor voltages.17

- Sampling Frequency-Based Approach: For improved performance in controlling capacitor voltages.17

- Redundancy Methods: Leveraging redundant switching states to balance capacitor voltages, as seen in efforts to improve balancing in FCMLIs where the maximally distended 8-level case may lack sufficient per-phase redundancy.17 A "segment redundancy method" has also been applied to enhance capacitor voltage.15

- Hybrid Modulation Schemes: These can be utilized to substantially reduce harmonic distortion and potentially aid in balancing.15

- Self-Balanced Voltage Capacitors: Some novel topologies are designed to achieve self-balanced capacitor voltage without additional circuits or complex algorithms, simplifying the control burden.19

- Research Opportunities: Future research includes developing Packed U-Cell 7-level (PUC7) inverters with inherent capacitor voltage balancing and gain.14 Another promising area is implementing soft charging for all switched capacitors to reduce losses and high inrush currents during charging, which are common issues in multi-capacitor switched capacitor (SC) MLIs.14 Long discharging periods and short charging windows for capacitors can lead to high voltage ripples, necessitating large and expensive capacitors 14, which further drives research into improved balancing. The continuous pursuit of inherent and soft balancing solutions aims to address the root causes of balancing problems and associated stresses, rather than just compensating for them externally.

4.4. Fault-Tolerant Control Strategies

Given that MLI designs often incorporate a large number of power semiconductor devices, system reliability can be impacted.23 Consequently, a significant area of research is dedicated to increasing dependability and including fault-tolerant qualities, either through software or hardware solutions.23 This is a critical practical consideration for industrial and grid applications where downtime is costly and safety is paramount.

- Common Faults: Faults can arise from uneven power distribution among PV cells or from damaged or bypassed H-bridge cells in topologies like CHB-MLIs, leading to power imbalance issues.11

- Strategies: Various fault-tolerant control methods have been proposed to ensure stable and continuous operation even under faulty conditions:

- Modified Level-Shifted PWM (LS-PWM): This method is proposed for CHMIs operating under faulty conditions, covering both open- and short-device fault cases. It can improve power distribution among H-bridge cells, allowing for balanced three-phase line-to-line voltages and currents with minimum harmonic distortion even with disabled power cells.23

- Neutral Voltage Modulation (NVM): NVM enables reliable operation even when DC source modules are damaged or operating at reduced voltage. It calculates the inverter's modulating signals based on the lowest and middle DC source voltages of each phase, then calculates and injects a common-mode voltage (CMV) modulating signal to reduce CMV and prevent overmodulation. This ensures balanced three-phase output with low THD, even with unbalanced DC sources.23

- Neutral-Shift Method: This method is proposed to produce balanced three-phase voltage and current, even with imbalanced phase-to-neutral voltages, by modifying PWM reference phase voltage angles and injecting a dynamic homopolar component.11

- Optimal Zero Sequence Injection Method (OZSI): Considered an advanced method to solve severe imbalance issues, generating balanced current.11

- Adaptive Control: Dynamically adjusting the inverter's modulating signals in response to voltage changes is a key aspect of advanced fault-tolerant control.23

- Fault-Tolerant Multilevel Inverter Drives: These are utilized in applications like Permanent Magnet Synchronous Motor (PMSM) drives to minimize performance degradation post-fault.24

The development of these strategies signifies a shift from simply detecting faults to actively mitigating their impact and ensuring continued, albeit potentially degraded, operation. This proactive approach to fault management is essential for high-availability systems and represents a significant advancement in MLI control, indicating that the technology is moving beyond basic functionality towards robust, real-world deployability.

4.5. Integration with Smart Grids and High Voltage Direct Current (HVDC) Systems

Multilevel inverters are becoming integral to the modernization of power infrastructure, particularly in the context of smart grids and High Voltage Direct Current (HVDC) transmission systems.

- Smart Grids: MLIs are pivotal in the transition to smart grids, especially with the increasing integration of Renewable Energy Systems (RES). This transition introduces control and protection challenges but also presents significant opportunities for MLI topology development and control research.13 The concept of "smart inverters" (S-INVs) based on MLIs is emerging, required for cost-effectively achieving power system operational performance under increasingly complex spatio-temporal power flow scenarios.25 Research also focuses on energy management systems for microgrids with multiple smart prosumers, optimizing load scheduling and communication sharing among hybrid renewable energy sources.13

- HVDC Applications: MLIs find applications in HVDC systems, which are crucial for long-distance bulk power transmission.4 Modular Multilevel Converters (MMCs), a specific type of MLI, are particularly relevant for HVDC transmission lines.7

- Challenges: The integration of RES into the grid introduces challenges such as maintaining frequency stability in power systems, which becomes more difficult as RES replaces conventional generators.25 There is an ongoing need for further evaluation of new topologies for their performance in grid-integrated applications.13 Modular MLIs, while suitable for large-scale RES integration, still face technical challenges in modeling, control, and protection within complex multi-terminal DC (MMC-MTDC) systems.13

The increasing need for "smartization based on data-centric operation" 25 and "energy management systems for microgrids" 13 suggests a strong convergence between power electronics (MLIs) and digital intelligence (IoT, control systems, data analytics). MLIs are evolving from mere power converters to "smart" devices capable of complex control, communication, and active participation in grid management functions. This is a significant trend, indicating that future MLI research will increasingly involve interdisciplinary approaches combining electrical engineering with computer science and data science to address the complexities of modern grids.

4.6. Efficiency and Power Quality Optimization

Despite the existing advantages of MLIs, ongoing research continues to relentlessly pursue further optimization of their efficiency and power quality (THD reduction).7 This continuous pursuit of "ideal" power conversion—approaching 100% efficiency and a pure sinusoidal output—is the ultimate, overarching goal of MLI research.

- Methods: Various approaches are being explored to achieve these goals:

- Reduced Switch Count Topologies: These inherently contribute to reduced conduction and switching losses, directly improving overall efficiency.14

- Advanced PWM Techniques: Sophisticated modulation methods are continuously refined to minimize switching losses and improve output waveforms.13

- Soft Charging of Capacitors: This is particularly important in multi-capacitor SC MLIs to reduce losses during charging and mitigate high inrush currents, thereby improving efficiency and reliability.14

- Resonant Charging: Investigation into resonant charging of switched capacitors in MLIs, particularly with fundamental frequency-modulated topologies, presents an opportunity for achieving highly efficient SC MLIs, as the low switching frequency allows for longer output voltage level holding times.14

- Fractional Capacitor Charging: This technique allows for the use of smaller, lower voltage, and less costly capacitors, potentially leading to more efficient inverters by optimizing component selection.14

- Optimization Algorithms: Algorithms like Particle Swarm Optimization (PSO) are used to determine precise switching angles for selective harmonic elimination and overall THD reduction.20

Every advancement, whether it is a new topology, a refined modulation strategy, or an improved balancing technique, ultimately aims to bring the inverter closer to the ideal of perfect DC-AC conversion. This is a continuous, iterative process driven by fundamental engineering principles and market demands for better performance.

5. Challenges and Future Directions

The rapid evolution and widespread adoption of multilevel inverters are accompanied by a set of persistent challenges that define current research frontiers and future development directions. These limitations often stem from the inherent trade-offs in MLI design, where optimizing one aspect can introduce complexities in another.

5.1. Current Limitations

- High Component Count and Cost: Classical MLI topologies generally require many components (switches, diodes, capacitors) as the number of output levels increases, leading to higher manufacturing costs and complex implementation.14 Each switch necessitates its own gate driver, adding to system complexity and cost.6 This is a core challenge, as the "ideal" MLI would achieve high performance with minimal components and simple control, but these are often conflicting objectives.

- Capacitor Voltage Balancing: This remains a significant challenge, especially for Flying Capacitor MLIs and some hybrid topologies. Maintaining stable capacitor voltages is complex and crucial for proper operation, as imbalance can lead to voltage distortion and system instability.14 This problem is exacerbated in multi-capacitor switched capacitor (SC) inverters.14

- High Inrush Currents: Some multi-capacitor SC inverters suffer from high inrush currents at startup, which can stress components and impact reliability.14

- Complexity with Multiple Voltage Sources: Asymmetric topologies, while beneficial for generating more levels with fewer switches, often require multiple isolated active voltage sources, which increases complexity and cost.14 Obtaining different voltage ratios from renewable energy systems can also be expensive.14

- Unbalanced Power Distribution: In some asymmetric topologies, power distribution and handling can be unbalanced due to voltage sources of different sizes, potentially leading to reliability issues due to uneven stress on components.14

- Computational Requirements: Certain topologies, particularly those designed for high voltage gain, may require increased computational capability for their control algorithms, especially as the number of carrier signals is tied to the voltage gain.14

- Voltage Ripples: Long discharging periods and short charging windows for capacitors can lead to high voltage ripples, necessitating the use of larger and more expensive capacitors to maintain power quality.14

- Intermittent Power Generation: The variable nature of renewable energy sources (e.g., PV) can cause voltage fluctuations, requiring robust MLI solutions for a steady and reliable energy supply.13

- Common-Mode Problems: In transformer-less systems, the lack of galvanic isolation can lead to common-mode current flow through parasitic capacitances, affecting system safety and performance.13

These limitations represent fundamental trade-offs in power electronics design. Future research will likely continue to navigate these compromises, seeking optimized solutions for specific application contexts rather than a single "universal" MLI. This implies a continuous cycle of innovation where new topologies or control methods address one set of problems but might introduce others, leading to further research.

5.2. Opportunities for Future Research and Development

The identified limitations concurrently highlight significant opportunities for future research and development in multilevel inverters, driving innovation towards more efficient, reliable, and intelligent power conversion systems.

- Novel Topologies:

- Combining Additive and Subtractive Operations in SC Topologies: Exploring the inclusion of subtractive operations of asymmetric voltage sources, in addition to additive operations, has the potential to generate more levels and eliminate the need for isolated DC sources, leading to high switch utilization.14

- Developing PUC7 with Inherent Voltage Balancing and Gain: There is an interesting opportunity to develop a 7-level packed U-cell (PUC7) inverter that incorporates inherent capacitor voltage balancing and voltage gain, potentially by combining existing concepts like sensor-less self-balancing PUC5 and reversed voltage sources.14

- Continued Reduced Switch Count and Hybrid Topologies: Ongoing development in these areas aims to achieve high power quality with reduced cost and complexity, pushing the boundaries of what is achievable with fewer components.14

- Advanced Control and Modulation:

- Implementing Soft Charging for All Switched Capacitors: This can lead to higher efficiency and the elimination of high-inrush capacitor currents, which are critical for improving reliability and lifespan.14

- Resonant Charging in Fundamental Frequency-Modulated Topologies: Investigating resonant charging of switched capacitors in MLIs that use fundamental frequency modulation presents an opportunity to achieve highly efficient SC MLIs, as the low switching frequency allows for longer output voltage level holding times.14

- Optimizing Capacitor Sizing and Cost: Research into charging capacitors to a fraction of the source voltage, along with voltage clamping or other control mechanisms, could lead to the use of smaller, lower voltage, and less costly capacitors, improving overall inverter efficiency and reducing physical footprint.14

- Efficient Model Predictive Current Control (MPCC) Algorithms: Further development of MPCC algorithms for MLIs can achieve quick dynamic response, low grid current ripples, effective capacitor voltage balancing, and contribute to reduced switch count.13

- Further Smartization of Inverters: This involves leveraging data-centric operation for improved power system performance, especially in scenarios with complex spatio-temporal power flow, indicating a convergence of power electronics with digital intelligence.25

- Reliability and Fault Tolerance:

- Addressing High Inrush Currents: Further development is needed to resolve this problem in multi-capacitor SC inverters, which impacts component stress and system reliability.14

- Improving Efficiency of Hybrid Topologies: While some hybrid topologies offer benefits, ongoing research can focus on optimizing their design to overcome drawbacks like the requirement for many different voltage sources.14

- Enhanced Fault-Tolerant Qualities: Continued research into software and hardware solutions is crucial to increase reliability, especially given the large number of devices in MLI designs.23

- Grid Integration and Applications:

- Energy Management Systems for Microgrids: Developing advanced techniques for managing energy in grid-linked microgrids with multiple smart prosumers is a key area for MLI application.13

- Evaluation of New Topologies: Further comprehensive evaluation of newly developed topologies and control approaches is essential to validate their performance in real-world grid-integrated applications.13

- Modular MLIs: Addressing technical challenges in modeling, control, and protection is vital for the widespread adoption of modular MLIs in large-scale RES integration.13

- RES-Enhanced Smart Grids: Continued research into control and protection challenges and opportunities is necessary as power systems evolve towards higher renewable energy penetration.13

The future of MLIs is multidisciplinary and driven by systemic energy needs. The pursuit of component reduction, efficiency gains, and intelligent control indicates that the fundamental challenges of cost, complexity, and efficiency remain central to MLI research. This implies a continuous push to refine these aspects, as even incremental improvements can yield substantial benefits for widespread adoption and economic viability, especially for large-scale applications. The increasing "smartization" of inverters points to a future where MLIs are not just passive converters but active, intelligent components within the power system, capable of complex control, communication, and participation in grid management functions.

6. Conclusion

Multilevel inverters represent a significant leap forward from conventional two-level inverters, fundamentally transforming DC-AC power conversion. Their ability to synthesize output voltages from multiple discrete levels enables the generation of near-sinusoidal waveforms with significantly reduced harmonic distortion, lower electromagnetic interference, and higher efficiency. These inherent advantages have positioned MLIs as indispensable components in critical sectors such as electric vehicles, large-scale renewable energy integration, industrial motor drives, and medium/high voltage grid-connected systems. They serve as a crucial bridge technology, facilitating grid modernization and the efficient management of diverse energy sources.

The field of multilevel inverters is characterized by dynamic and continuous innovation. Research is actively pursuing novel topologies, with a strong emphasis on Reduced Switch Count (RSC) and hybrid configurations, particularly asymmetrical designs, to overcome the inherent trade-offs between performance and component complexity. Concurrently, advancements in control and modulation strategies, including Selective Harmonic Elimination (SHE), Space Vector Modulation (SVM), and Model Predictive Control (MPC), are pushing the boundaries of power quality and efficiency. Enhanced capacitor voltage balancing techniques and robust fault-tolerant control strategies are vital areas of focus, addressing the reliability challenges posed by the increasing number of devices in MLI designs. Furthermore, the integration of MLIs into smart grids and HVDC systems highlights their strategic importance in the evolving global energy infrastructure.

While challenges persist, notably concerning component count, cost, capacitor voltage balancing, and the complexities of integrating with dynamic grids, the ongoing research and development efforts are systematically addressing these limitations. The future of MLIs points towards increasingly "smart," integrated, and self-optimizing solutions, leveraging computational intelligence and interdisciplinary approaches to meet the stringent demands of future power systems. The continuous innovation in MLI design and control is therefore crucial for building efficient, reliable, and sustainable power systems that underpin the global energy transition.

Works cited

- An Extensive Review of Multilevel Inverters Based on Their Multifaceted Structural Configuration, Triggering Methods and Applications - MDPI, accessed July 21, 2025, https://www.mdpi.com/2079-9292/9/3/433

- Cascaded H-Bridge Multilevel Inverter - CORE, accessed July 21, 2025, https://core.ac.uk/download/pdf/11783457.pdf

- Cascaded H-bridge multilevel inverter employing bidirectional switches | Request PDF, accessed July 21, 2025, https://www.researchgate.net/publication/224204274_Cascaded_H-bridge_multilevel_inverter_employing_bidirectional_switches

- A New Multilevel Inverter Topology With Reduce Switch Count - ResearchGate, accessed July 21, 2025, https://www.researchgate.net/publication/332860667_A_New_Multilevel_Inverter_Topology_With_Reduce_Switch_Count

- DIODE CLAMPED MULTILEVEL INVERTER ... - DigitalXplore, accessed July 21, 2025, https://www.digitalxplore.org/up_proc/pdf/332-1514884450185-189.pdf

- Design rules for diode-clamped multilevel inverters used in medium-voltage applications, accessed July 21, 2025, https://www.researchgate.net/publication/4024510_Design_rules_for_diode-clamped_multilevel_inverters_used_in_medium-voltage_applications

- (PDF) A Review on Multilevel Inverter Topologies - ResearchGate, accessed July 21, 2025, https://www.researchgate.net/publication/358267935_A_Review_on_Multilevel_Inverter_Topologies

- Multilevel inverter applications for electric vehicle drives | Request ..., accessed July 21, 2025, https://www.researchgate.net/publication/350586092_Multilevel_inverter_applications_for_electric_vehicle_drives

- (PDF) Review of Multilevel Inverters for PV Energy System ..., accessed July 21, 2025, https://www.researchgate.net/publication/350020711_Review_of_Multilevel_Inverters_for_PV_Energy_System_Applications

- Multilevel Inverters for Electric Vehicle Applications - UNT Digital Library, accessed July 21, 2025, https://digital.library.unt.edu/ark:/67531/metadc667233/

- (PDF) Control of a Cascaded H-bridge Multilevel Inverter with Failed ..., accessed July 21, 2025, https://www.researchgate.net/publication/335347581_Control_of_a_Cascaded_H-bridge_Multilevel_Inverter_with_Failed_Cells_for_Grid-Connected_Application

- A new multilevel inverter with alleviated number of switches for ..., accessed July 21, 2025, https://researchers.mq.edu.au/en/publications/a-new-multilevel-inverter-with-alleviated-number-of-switches-for-

- A Comprehensive Review on Multilevel Inverters for Grid-Tied ..., accessed July 21, 2025, https://www.mdpi.com/1996-1073/15/17/6315

- A structural review on reduced switch count and hybrid ... - Frontiers, accessed July 21, 2025, https://www.frontiersin.org/journals/energy-research/articles/10.3389/fenrg.2024.1396149/full

- Extended operation of flying capacitor multilevel inverters | Request PDF - ResearchGate, accessed July 21, 2025, https://www.researchgate.net/publication/4099502_Extended_operation_of_flying_capacitor_multilevel_inverters

- Flying capacitors multilevel inverter: architecture, control and active ..., accessed July 21, 2025, https://www.e3s-conferences.org/articles/e3sconf/pdf/2022/03/e3sconf_icegc2022_00039.pdf

- Capacitor Voltage Balancing Schemes in Flying Capacitor Multilevel Inverters | Request PDF - ResearchGate, accessed July 21, 2025, https://www.researchgate.net/publication/224289804_Capacitor_Voltage_Balancing_Schemes_in_Flying_Capacitor_Multilevel_Inverters

- Extended operation of flying capacitor multilevel inverters | Request ..., accessed July 21, 2025, https://www.researchgate.net/publication/3280832_Extended_operation_of_flying_capacitor_multilevel_inverters

- A novel symmetrical multilevel inverter topology with reduced ..., accessed July 21, 2025, https://www.researchgate.net/publication/308495759_A_novel_symmetrical_multilevel_inverter_topology_with_reduced_switching_devices_using_different_PWM_techniques

- Utilizing the Optimization Algorithm in Cascaded H-Bridge Multilevel Inverter, accessed July 21, 2025, https://www.researchgate.net/publication/279957921_Utilizing_the_Optimization_Algorithm_in_Cascaded_H-Bridge_Multilevel_Inverter

- [2104.09643] Selective Harmonic Elimination in a Cascaded H-Bridge Multilevel Inverter fed by High-Frequency Isolated DC-DC Converter - arXiv, accessed July 21, 2025, https://arxiv.org/abs/2104.09643

- (PDF) A Review Paper on the Elimination of Low-Order Harmonics ..., accessed July 21, 2025, https://www.researchgate.net/publication/345490062_A_Review_Paper_on_the_Elimination_of_Low-Order_Harmonics_in_Multilevel_Inverters_Using_Different_Modulation_Techniques

- A Modified Level-Shifted PWM Strategy for Fault-Tolerant Cascaded ..., accessed July 21, 2025, https://www.researchgate.net/publication/299499834_A_Modified_Level-Shifted_PWM_Strategy_for_Fault-Tolerant_Cascaded_Multilevel_Inverters_With_Improved_Power_Distribution

- An improved space vector PWM control algorithm for multilevel inverters - ResearchGate, accessed July 21, 2025, https://www.researchgate.net/publication/4114944_An_improved_space_vector_PWM_control_algorithm_for_multilevel_inverters

- Challenges in Smartizing Operational Management of Functionally-Smart Inverters for Distributed Energy Resources: A Review on Machine Learning Aspects - MDPI, accessed July 21, 2025, https://www.mdpi.com/1996-1073/16/3/1330