Neutral-Point Clamped (NPC) / Diode Clamped

- Why considered: produces multiple voltage levels from a single DC bus.

- Why not used here:

  - Neutral-point (midpoint of DC bus) voltage drifts → unbalanced capacitor voltages → unreliable under varying motor load.

  - If one switch/diode fails, the whole inverter leg is affected — you cannot simply remove one module and keep running. No modularity, poor fault isolation.

2. Active Neutral-Point Clamped (ANPC)

- Why considered: improves current sharing and efficiency compared to NPC.

- Why not used here:

  - Same neutral-point balancing problem remains.

  - Still tied to a single DC link → if fault happens, whole inverter is compromised.

  - Can’t “hot-swap” modules → fails modularity requirement.

3. Flying Capacitor Multilevel (FCML)

- Why considered: produces multiple levels without isolated DC sources.

- Why not used here:

  - Needs large floating capacitors → bulky and not practical for EV where space/weight are premium.

  - Capacitor voltage balancing is very sensitive to load changes (motor acceleration, regen braking).

  - If one capacitor fails, entire leg performance collapses; again, no easy modular replacement.

4. Modular Multilevel Converter (MMC)

- Why considered: highly modular, used in HVDC transmission.

- Why not used here:

  - Each phase arm needs dozens of submodules + large arm inductors. This makes it heavy and bulky — not suitable for EV powertrain.

  - Designed for very high voltage, low frequency (grid) — not optimal for fast-switching motor drive (kHz range).

  - High circulating currents → extra loss in compact EV inverter.

5. Matrix Converter (AC–AC direct)

- Why considered: no DC link capacitor, compact.

- Why not used here:

  - Needs 9 bidirectional switches with perfect commutation → very sensitive to switching faults.

  - No modularity: if one switch fails, whole converter fails.

  - Hard to integrate with EV battery (DC source), since it is meant for AC–AC.

6. Z-Source / Quasi-Z-Source Inverter

- Why considered: can boost DC voltage and ride-through dips.

- Why not used here:

  - The impedance network (inductors + capacitors) adds bulk → not modular, just one big stage.

  - If fault occurs in the inverter, the whole system must shut down; you cannot bypass individual units.

  - Good for single-inverter robustness, but doesn’t support the “replaceable module” goal.

6. 3 phase inverter

- to put voltages in series you must series-connect corresponding phase outputs, not whole 3-phase inverter outputs. that requires full galvanic isolation between inverter neutrals — messy and unsafe.

- full 3-phase boxes contain their own star/neutral reference; series-stacking them without careful isolation and control will create circulating currents, ground faults and unpredictable voltages.

- balancing the DC-link voltages becomes harder if whole 3-phase units are stacked.

Final justification:
All the above topologies either:

- depend on a single DC link (NPC, ANPC, Z-source),

- need large bulky passive components (Flying-capacitor, MMC, Z-source),

- or lose the entire inverter if one switch fails (Matrix, NPC).

Only Cascaded H-Bridge (CHB) (or half-bridge variant) allows independent DC modules, easy bypass of faulty cells, and true modularity — which is exactly your project’s aim.