INDIVIDUAL H-BRIDGE OPERATION:

Each H-bridge can be represented by the table:

| H-Bridge Output | S1 | S2 | S3 | S4 | Output Description |
| --- | --- | --- | --- | --- | --- |
| +Vdc | 1 | 1 | 0 | 0 | Diagonal switches ON (positive terminal connected to +Vdc) |
| 0 | 0 | 1 | 1 | 0 | Bottom switches conduction path (output shorted through middle switches) |
| −Vdc | 0 | 0 | 1 | 1 | Diagonal switches ON (output connected to −Vdc) |

These three modes give a clean transition between voltage levels and prevent shoot-through.

STRUCTURE OF 5-LEVEL CHB INVERTER:

Each phase leg consists of two single-phase H-bridges connected in series:

Each H-bridge has 4 switches (say S1–S4 for Bridge 1, S5–S8 for Bridge 2).

- Each H-bridge produces three possible output voltages:

  - +Vdc ((S1,S2)diagonal switches ON)

  - 0 (bottom switches ON)

  - −Vdc ((S3,S4)diagonal switches ON)

When the two H-bridges’ outputs add up in series, the total output voltage has 5 levels:

Vout={−2Vdc,−Vdc,0,+Vdc,+2Vdc}

- CASCADED COMBINATION (TWO H-BRIDGES)

Now, by combining Bridge-1 and Bridge-2, we can form the 5 voltage levels as follows:

| Desired Output | Bridge 1 Output | Bridge 2 Output | Bridge 1 (S1–S4) | Bridge 2 (S5–S8) |
| --- | --- | --- | --- | --- |
| +2Vdc | +Vdc | +Vdc | 1 1 0 0 | 1 1 0 0 |
| +Vdc | +Vdc | 0 | 1 1 0 0 | 0 1 1 0 |
| 0 | 0 | 0 | 0 1 1 0 | 0 1 1 0 |
| −Vdc | −Vdc | 0 | 0 0 1 1 | 0 1 1 0 |
| −2Vdc | −Vdc | −Vdc | 0 0 1 1 | 0 0 1 1 |

Switching Logic Summary

- The sign of the reference level L decides which bridges are positive or negative.

- The magnitude of L decides how many bridges should output +Vdc or −Vdc.

For example:

- If L = 2: Both bridges are +Vdc → +2Vdc total

- If L = 1: One bridge is +Vdc, one is 0 → +Vdc total

- If L = 0: Both are 0 → 0V total

- If L = −1: One is −Vdc, one is 0 → −Vdc total

- If L = −2: Both are −Vdc → −2Vdc total

STRUCTURE OF 7-LEVEL CHB INVERTER:

Each phase leg consists of three single-phase H-bridges connected in series:

Each H-bridge has 4 switches (say S1–S4 for Bridge 1, S5–S8 for Bridge 2,S9-S12 for Bridge 3).

- Each H-bridge produces three possible output voltages:

  - +Vdc ((S1,S2)diagonal switches ON)

  - 0 (bottom switches ON)

  - −Vdc ((S3,S4)diagonal switches ON)

When the three H-bridges’ outputs add up in series, the total output voltage has 7 levels:

Vout={-3Vdc,−2Vdc,−Vdc,0,+Vdc,+2Vdc,+3Vdc}

- CASCADED COMBINATION (THREE H-BRIDGES)

Now, by combining Bridge-1 and Bridge-2 and Bridge-3, we can form the 7 voltage levels as follows:

Now the canonical gate-patterns for level L:

| Level | Module 1 | Module 2 | Module 3 | G1 G2 G3 G4 | G5 G6 G7 G8 | G9 G10 G11 G12 |
| --- | --- | --- | --- | --- | --- | --- |
| +3 | +1 | +1 | +1 | 1 0 0 1 | 1 0 0 1 | 1 0 0 1 |
| +2 | +1 | +1 | 0 | 1 0 0 1 | 1 0 0 1 | 1 0 1 0 |
| +1 | +1 | 0 | 0 | 1 0 0 1 | 1 0 1 0 | 1 0 1 0 |
| 0 | 0 | 0 | 0 | 1 0 1 0 | 1 0 1 0 | 1 0 1 0 |
| -1 | -1 | 0 | 0 | 0 1 1 0 | 1 0 1 0 | 1 0 1 0 |
| -2 | -1 | -1 | 0 | 0 1 1 0 | 0 1 1 0 | 1 0 1 0 |
| -3 | -1 | -1 | -1 | 0 1 1 0 | 0 1 1 0 | 0 1 1 0 |

Notes:

- The table uses the canonical decomposition [+1,+1,0] for +2 etc. You can choose alternate decompositions to balance switching losses (e.g., sometimes use [+1,0,+1]).

- The chosen 0 state is Z+ for each module (all top switches ON). If you prefer Z− (all bottom ON), replace module zero pattern with [0,1,0,1] consistently.

STRUCTURE OF 9-LEVEL CHB INVERTER:

Each phase leg consists of four single-phase H-bridges connected in series:

Each H-bridge has 4 switches (say S1–S4 for Bridge 1, S5–S8 for Bridge 2,S9-S12 for Bridge 3, S13-S16 for Bridge 4).

- Each H-bridge produces three possible output voltages:

  - +Vdc ((S1,S2)diagonal switches ON)

  - 0 (bottom switches ON)

  - −Vdc ((S3,S4)diagonal switches ON)

When the three H-bridges’ outputs add up in series, the total output voltage has 7 levels:

Vout={-4Vdc, -3Vdc,−2Vdc,−Vdc,0,+Vdc,+2Vdc,+3Vdc, +4Vdc}

Truth table (canonical mapping)

| Level | Module 1 | Module 2 | Module 3 | Module 4 | G1 G2 G3 G4 | G5 G6 G7 G8 | G9 G10 G11 G12 | G13 G14 G15 G16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +4 | +1 | +1 | +1 | +1 | 1 0 0 1 | 1 0 0 1 | 1 0 0 1 | 1 0 0 1 |
| +3 | +1 | +1 | +1 | 0 | 1 0 0 1 | 1 0 0 1 | 1 0 0 1 | 1 0 1 0 |
| +2 | +1 | +1 | 0 | 0 | 1 0 0 1 | 1 0 0 1 | 1 0 1 0 | 1 0 1 0 |
| +1 | +1 | 0 | 0 | 0 | 1 0 0 1 | 1 0 1 0 | 1 0 1 0 | 1 0 1 0 |
| 0 | 0 | 0 | 0 | 0 | 1 0 1 0 | 1 0 1 0 | 1 0 1 0 | 1 0 1 0 |
| -1 | -1 | 0 | 0 | 0 | 0 1 1 0 | 1 0 1 0 | 1 0 1 0 | 1 0 1 0 |
| -2 | -1 | -1 | 0 | 0 | 0 1 1 0 | 0 1 1 0 | 1 0 1 0 | 1 0 1 0 |
| -3 | -1 | -1 | -1 | 0 | 0 1 1 0 | 0 1 1 0 | 0 1 1 0 | 1 0 1 0 |
| -4 | -1 | -1 | -1 | -1 | 0 1 1 0 | 0 1 1 0 | 0 1 1 0 | 0 1 1 0 |

How to read a row: e.g. for L = +2, module outputs are [+1, +1, 0, 0].

- Module1 gates G1..G4 = 1 0 0 1 (module1 = +V)

- Module2 gates G5..G8 = 1 0 0 1 (module2 = +V)

- Module3 gates G9..G12 = 1 0 1 0 (module3 = 0 → Z+)

- Module4 gates G13..G16 = 1 0 1 0 (module4 = 0 → Z+)